Much of the hardware in astronauts’ inventory of custom tools are actually modified versions of common tools1.

- sockets

- socket extender bars

- handheld temperature sensor

- handheld infrared camera

Specialty Tools

Pistol Grip Tool (PGT)

Half drill-driver,half ratcheting torque wrench

Tool Bag

Box which folds up and can be stored in the shuttle’s mid-deck locker that astronauts can fill up with tools for any task.

Handheld Paddle

About the size of a ping-pong paddle which holds the patching material for repairing cracks.

Modified Mini-Workstation Tool Stowage Caddy (MMWS)

Tool belt designed to easy space use. It wraps around the astronaut and has retractable tethers with clips that attach to the tools. Prevents tools from floating away.

Tools used to service NASA’s Hubble Space Telescope

- (Right) Pistol Grip Tool

- (Foreground) Card extraction and insertion tools for electronic cards

- (Top Center) Pistol Grip Tool - plastic version

- (Center Left) Bit Caddy

Mini Power Tool (a.k.a Drill)

Rate Sensing Units Changeout Tool

Special tool to remove old insulation from the telescope so that new protective thermal insulation could be installed. Can reach inside the telescope without entering it and capture and restrain a sensor unit by pulling a trigger.

Battery Extractor

EVA Tools and Equipment Reference Book

http://ssco.gsfc.nasa.gov/documents/5_EVA_Tools_and_Equipment_Reference_Book_Nov_83.pdf

Modified Mini-Workstation Tool Stowage Caddy (MMWS)

http://www.cram.com/flashcards/eva-small-tools-1320426

Modular Baseplate Assembly

mounting point for MMWS system components

Modular Gimbal Assembly

Retracting End Effector Assembly

capability to lock onto a handrail, tether point, or tether loops

Swing Arm Assembly

Contains two bayonet receptacles per swing arm. Each bayonet receptacle has a slide lock to secure the equipment in the receptacle. Allows stowing hardware behind the EMU while translating and then swing the hardware out in front.